# Data Dictionary for North Coast Puerto Rico Shoreline Analysis

This section includes the definition of the main data concepts used in the “North Coast Puerto Rico Shoreline Analysis 2022-2023” dataset in eleven (11) municipalities at Puerto Rico (Rincón, Aguadilla, Isabela, Hatillo, Arecibo, Dorado, Vega Baja, Carolina, Loíza, Río Grande, and Luquillo).

The table below describes the attributes for all feature classes in the file geodatabase (.gdb) and shapefiles (.shp) included in this data release. The cartographic information is organized into eight main data types (shorelines, back beach, beach edges, beach polygons, lost transects from 2018 to 2022-2023, beach transects 5 m, beach transects 2018 vs 2022-2023 and shoreline displacement).

Please note the field names in the attribute table of the shapefiles are abbreviated. The tables -below detail the complete field name visible in the geodatabase feature classes in the ArcGIS Pro Map Package file.

## Shorelines

## Layer: North_Coast_Puerto_Rico_Shoreline_2022_2023

| Attribute Label | Attribute Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri. |
| Shape* | The "Shape" field indicates the layer's geometry (Polyline Z). This feature geometry and coordinates is automatically generated by Esri. |
| Coast | Orientation of the coastal municipality: north, south, east, or west. |
| Municipality | Name of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón, Aguadilla, Isabela, Hatillo, Arecibo, Dorado, Vega Baja, Carolina, Loíza, Río Grande, and Luquillo. |
| Year_Photo | Time period during which the high-resolution images were taken (in 'Month_YYYY' format). |
| Coastal_Type | Characterizes the geomorphic and geological diversity of the Puerto Rican shoreline during the digitization process. The main coastal types include: 1) Alluvial (shorelines mainly associated with river mouths and minor alluvial systems); 2) Anthropogenic structures (coastal protective hard structures and/or residential structures in the shoreline); 3) Beach (shoreline composed of unconsolidated sand and/or gravel material); 4) Rocky (shoreline segments composed of consolidated geological material). This coastal type employs the wet/dry zone as the primary indicator for digitization in rocky shorelines; 5) Vegetation (for this shoreline, the maximum extent of the closed canopy was used as an indicator). Additionally, two (2) complementary categories were incorporated to facilitate the digitization workflow: 6) Not Surveyed (applied to segments not present in the aerial imagery under study; these shorelines were validated with field data); and 7) Undefined (used to categorize shorelines that cannot be classified into the main coastal types). |
| Wetline_Bio_Natural_Barrier | Represents the presence of biological natural barriers located in direct contact with the shoreline type previously classified by the GIS analyst. This structure is identified because it performs important ecological and physical functions in coastal protection. The main categories included in this field are: Dunes no vegetation, Dune with vegetation, Mangroves, No presence, Not Surveyed, Other wetlands, Undefined, Vegetation, Corals/coral reef, or Seagrass beds. |
| Wetline_Geo_Natural_Barrier | Represents the presence of geological natural barriers located in direct contact with the shoreline type previously classified by the GIS analyst. The main categories included in this field are: Beachrock (Holocene consolidated sand-gravel deposits), Eolianite (lithified coastal dune deposits), Multiple, No presence, Not Surveyed, Other, Rocks (Igneous, metamorphic, and sedimentary), Undefined, Cay/Islet, Sand bar/beach sand-gravel deposits, or Sand bar/sand deposits. Undefined (used to categorize geological natural barriers that cannot be classified into the main types). |
| Nearshore_Bio_Natural_Barrier | Represents the presence of biological natural barriers located a kilometer (1.0 km) from the shoreline. This structure is identified because it performs important ecological, oceanographic, and physical functions in coastal protection. The main categories included in this field are: Mangroves, No presence, Not Surveyed, Undefined, Seagrass beds, Multiple, Vegetation, Coral reef, or Corals/Coral reef. Undefined (used to categorize natural barriers that cannot be classified into the main types). These additional classifications are intended solely for internal processing and quality control and do not represent definitive natural barrier types except for the “undefined” category. These natural structures were identified through photointerpretation. They were subsequently validated through field inspections and using the following cartographic source: National Oceanic and Atmospheric Administration National Centers for Coastal Ocean Science, 2017, Benthic Habitat Mapping in Puerto Rico and the U.S. Virgin Islands for a Baseline Inventory: https://coastalscience.noaa.gov/project/benthic-habitat-mapping-puerto-rico-virgin-islands/ |
| Nearshore_Geo_Natural_Barrier | Represents the presence of geological natural barriers located a kilometer (1.0 km) from the shoreline. The main categories included in this field are: Beachrock (Holocene consolidated sand-gravel deposits), Eolianite (lithified coastal dune deposits), Multiple, No presence, Not Surveyed, Other, Rocks (Igneous, metamorphic, and sedimentary), Undefined, Cay/Islet, or Sand bar/beach sand-gravel deposits. Undefined (used to categorize geological natural barriers that cannot be classified into the main types). |
| Natural_Barrier_Coast_Beach | Represents the presence of natural structures on the coast within 500 m. These structures are Dunes no vegetation, Dunes with vegetation, Mangroves, Other wetlands, Vegetation, Multiple, and No presence. Additionally, two complementary categories were incorporated to facilitate the digitization workflow: Not Surveyed (applied to segments not present in the aerial imagery under study; these natural barriers were validated with field data); and Undefined (used to categorize natural barriers that cannot be classified into the main 7 categories). These natural structures were identified through photointerpretation. They were subsequently validated through field inspections and using the following cartographic source: National Oceanic and Atmospheric Administration National Centers for Coastal Ocean Science, 2017, Benthic Habitat Mapping in Puerto Rico and the U.S. Virgin Islands for a Baseline Inventory: https://coastalscience.noaa.gov/project/benthic-habitat-mapping-puerto-rico-virgin-islands/ |

| Hard_Structure | Formal and/or informal structures on the coast: Boat Ramp, Breakwater, Bridge, Bulkhead, Docks, Groynes, Jetties, Multiple, No presence, Not Surveyed, Riprap, Seawall, Structure with damage, Undefined, Informal Structures, or Structures residential/road/commercial. These additional classifications are intended solely for internal processing and quality control and do not represent definitive hard structure types except for the “undefined” category. |
| --- | --- |
| Location_Direction_Hard_Structure | Location of the coastal structure: Multiple, Nearshore parallel, No presence, Not Surveyed, Offshore, Shoreline parallel, Shoreline perpendicular, Undefined, Nearshore parallel, Nearshore perpendicular, Backshore parallel, Backshore perpendicular, Foreshore perpendicular, Foreshore parallel, or Shoreline-Foreshore parallel. |
| Infrastructure_1km_CoastalZone | Represents the type of infrastructure present within a 1,000-meter-wide strip inland from the shoreline: Commercial, Multiple, No presence, Other, Residential, or Roads. |
| Shape_Length* | Positive real numbers that are automatically generated by the GIS software to define the length of the polyline feature. |

## Back Beach

## Layer: North_Coast_Puerto_Rico_Back_Beach_2022_2023

| Attribute_Label | Attribute_Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri. |
| Shape* | The "Shape" field indicates the layer's geometry (Polyline Z). This feature geometry and coordinates is automatically generated by Esri. |
| Coast | Orientation of the coastal municipality: north, south, east, or west. |
| Municipality | Name of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón, Aguadilla, Isabela, Hatillo, Arecibo, Dorado, Vega Baja, Carolina, Loíza, Río Grande, and Luquillo. |
| Year_Photo | Time period during which the high-resolution images were taken (in 'Month_YYYY' format). |
| Type_Backbeach | Represents the zone of the subaerial beach that delineates the physical boundary in an aerial image between the unconsolidated sand, coastal vegetation, rocks, coastal cliffs, coastal bluffs, compacted sand, dirt roads, dunes, and/or structures of anthropogenic origin. The back beach categories in Puerto Rico include (1) Compacted sand/dirt road/unpaved road, (2) Dunes, (3) Infrastructure, (4) Mitigation Hard Structures, (5) Riverine/Bar, (6) Road, (7) Rock, (8) Vegetation, (9) Undefined, and (10) Other. The visual limit of these categories in an aerial image was used as a proxy for the back beach indicator, representing the limit of the subaerial beach. |
| Shape_Length* | Positive real numbers that are automatically generated by the GIS software to define the length of the polyline feature. |

## Beach Edges

## Layer: North_Coast_Puerto_Rico_Beach_Edges_2022_2023

| Attribute_Label | Attribute_Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri. |
| Shape* | The "Shape" field indicates the layer's geometry (Polyline X). This feature geometry and coordinates is automatically generated by Esri. |
| Type | The principal edge categories include (1) Tombolo (Accumulation of unconsolidated material along the coast that forms an isthmus connecting an island to a headland or another smaller landmass), (2) Infrastructure (Any physical element of anthropogenic origin, generally residential and/or commercial structures. This category excludes coastal mitigation structures and roads), (3) Rock (Sedimentary, igneous and/or metamorphic rocks), (4) Sand, (5) Vegetation, (6) Mitigation Hard Structure, (7) Road (Asphalted roadway), (8) Riverine/Bar (Coastal sediments transported by fluvial systems to the mouths of rivers and streams), (9) Dune (Unconsolidated sand deposits transported by wind in the backshore area), and (10) Other. Additionally, a complementary beach edge category was incorporated to facilitate the digitization workflow: (11) Undefined (used to categorize beach edges that cannot be classified into the main coastal types). This additional classification is intended solely for internal processing and quality control and do not represent definitive coastal types except for the “undefined” category. |
| Shape_Length* | Positive real numbers that are automatically generated by GIS software to define the length of the polyline feature. |
| Year_Photo | Time period during which the high-resolution images were taken (in 'Month_YYYY' format). |

## Beach Polygons

## Layer: North_Coast_Puerto_Rico_Beach_Polygon_2022_2023

| Attribute_Label | Attribute_Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri |
| Shape* | The "Shape" field indicates the layer's geometry (polygon Z). This feature geometry and coordinates is automatically generated by Esri. |
| Shape_Length* | Positive real numbers that are automatically generated by GIS software to define the length of the polygon feature. |
| Shape_Area* | Positive real numbers that are automatically generated by the GIS software to define the area of the polyline feature. |

## Beach Transects 5m

## Layer: North_Coast_Puerto_Rico_Beach_Width_Transects_5m_Interval_2022_2023

| Attribute_Label | Attribute_Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri. |
| Shape* | The "Shape" field indicates the layer's geometry (Polyline). This feature geometry and coordinates is automatically generated by Esri. |
| Municipality | Name code of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón (RIN), Aguadilla (AGU), Isabela (ISA), Hatillo (HAT), Arecibo (ARE), Dorado (DOR), Vega Baja (VBA), Carolina (CAR), Loíza (LOI), Río Grande (RGR), and Luquillo (LUQ). |
| Unique_ID_Municipality | Represents transects with a unique code for each municipality (above). For example: Rincón (RIN), Aguadilla (AGU), Isabela (ISA), Hatillo (HAT), Arecibo (ARE), Dorado (DOR), Vega Baja (VBA), Carolina (CAR), Loíza (LOI), Río Grande (RGR), and Luquillo (LUQ). Each municipality name code is followed by the unique number for each transect. Each transect is perpendicular to the shoreline and is spaced 5 meters apart. |
| NSM_BW_2022_2023 | Indicates the linear extent of the beach transect from the back beach to the shoreline for the 2022–2023 study period. Each transect is perpendicular to the maximum length of the beach and is spaced 5 m apart. |
| Shape_Length* | Positive real numbers that are automatically generated by GIS software to define the length of the polyline feature. |

## Beach Transects

## Layer: North_Coast_Puerto_Rico_Beach_Erosion_Accretion_Transects_2018_2022_2023

| Attribute_Label | Attribute_Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri, |
| Shape* | The "Shape" field indicates the layer's geometry (Polyline). This feature geometry and coordinates is automatically generated by Esri. |
| Town | Name of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón, Aguadilla, Isabela, Hatillo, Arecibo, Dorado, Vega Baja, Carolina, Loíza, Río Grande, and Luquillo. |
| Municipality_abbreviation | Abbreviation of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón (RIN), Aguadilla (AGU), Isabela (ISA), Hatillo (HAT), Arecibo (ARE), Dorado (DOR), Vega Baja (VBA), Carolina (CAR), Loíza (LOI), Río Grande (RGR), and Luquillo (LUQ). |
| Unique_ID_Municipality | Represents transects with a unique code for each municipality (above). For example: Rincón (RIN), Aguadilla (AGU), Isabela (ISA), Hatillo (HAT), Arecibo (ARE), Dorado (DOR), Vega Baja (VBA), Carolina (CAR), Loíza (LOI), Río Grande (RGR), and Luquillo (LUQ). Each municipality name code is followed by the unique number for each transect. |
| North_Azimuth_360 | Azimuth direction of the transect. The coastal transect orientation is measured in degrees clockwise from north on an azimuth circle. An azimuth circle consists of 360 degrees. Ninety degrees corresponds to east, 180 degrees is south, 270 degrees is west, and 360 degrees and 0 degrees mark north. |
| NSM_BW_2022_2023 | Indicates the linear extent of the beach transect from the back beach to the shoreline for the 2022–2023 study period. |
| Change_July_2018_2022_2023 | This field represents the change in meters in beach width from July 2018 to the 2022–2023 study period. This field was calculated by subtracting the width of the most recent transect from that of the previous one. |
| Erosion_Accretion_July18_22_23 | Erosion-accretion status of the subaerial beach. |
| Shape_Length* | Positive real numbers that are automatically generated by Esri to define the length of the polyline feature. |

## Beach Displacement

## Layer: North_Coast_Puerto_Rico_Beach_Displacement_2018_vs_2022_2023

| Attribute_Label | Attribute_Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri. |
| Shape* | The "Shape" field indicates the layer's geometry (Polyline). This feature geometry and coordinates is automatically generated by Esri. |
| Municipality | Name of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón, Aguadilla, Isabela, Hatillo, Arecibo, Dorado, Vega Baja, Carolina, Loíza, Río Grande, and Luquillo. |
| Shoreline_displacement | Shoreline Displacement Analysis: The shoreline displacement was evaluated across four representative domains based on the photointerpretation of imagery. The domains are: 1- Inland Displacement: showed the landward movement of the shoreline, measured in meters, from sea to land. 2- Seaward Displacement: showed the outward movement of the shoreline toward the sea. 3- Structure Displacement: showed the seaward displacement caused by coastal structures. These structures may include formal mitigation structures, informal mitigation structures, or residential/commercial infrastructure. 4- Vegetation Displacement: Showed shoreline displacement caused by changes in the vegetation canopy. For this study, the limit of coastal vegetation and forests was digitized to serve as an indicator of the shoreline. |
| Unique_ID | Represents transects with a unique code for each municipality: Rincón (RIN), Aguadilla (AGU), Isabela (ISA), Hatillo (HAT), Arecibo (ARE), Dorado (DOR), Vega Baja (VBA), Carolina (CAR), Loíza (LOI), Río Grande (RGR), and Luquillo (LUQ). Each municipality name code is followed by the unique number for each transect. Each transect is perpendicular to the shoreline and is spaced 5 meters apart. |
| Shape_Length* | Positive real numbers that are automatically generated by Esri to define the length of the polyline feature. |

## Lost Transects from 2018 to 2022-2023

## Layer: North_Coast_Puerto_Rico_Lost_Transects_2018_2022_2023

| Attribute_Label | Attribute_Definition |
| --- | --- |
| OBJECTID* | Sequential unique whole numbers that are automatically generated by Esri. |
| Shape* | The "Shape" field indicates the layer's geometry (line). This feature geometry and coordinates is automatically generated by Esri. |
| Municipality | Name of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón, Aguadilla, Isabela, Hatillo, Arecibo, Dorado, Vega Baja, Carolina, Loíza, Río Grande, and Luquillo. |
| Unique_ID_Municipality | Represents transects with a unique code for each municipality: Rincón (RIN), Aguadilla (AGU), Isabela (ISA), Hatillo (HAT), Arecibo (ARE), Dorado (DOR), Vega Baja (VBA), Carolina (CAR), Loíza (LOI), Río Grande (RGR), and Luquillo (LUQ). Each municipality name code is followed by the unique number for each transect. Each transect is perpendicular to the shoreline and is spaced 5 meters apart. |
| Municipality_abbreviation | Abbreviation of the coastal municipality in the northwest and north coasts of Puerto Rico: Rincón (RIN), Aguadilla (AGU), Isabela (ISA), Hatillo (HAT), Arecibo (ARE), Dorado (DOR), Vega Baja (VBA), Carolina (CAR), Loíza (LOI), Río Grande (RGR), and Luquillo (LUQ). |