# Data Dictionary for Grain-Size Tables

The table below describes the attributes (data columns) for the grain-size data tables presented in this report.

The metadata for the grain-size data are not complete if they are not distributed with this document.

| Attribute_Label | Attribute_Definition |
| --- | --- |
| Core ID | Core identification number |
| Depth (cm) | Sample depth interval, in centimeters |
| Median Depth (cm) | Midpoint of sample depth, in centimeters |
| Sediment Texture (Folk, 1954) | Physical description of sediment textural group - describes the dominant grain size class of the sample (after Folk, 1954): Sand, Clayey Sand, Muddy Sand, Silty Sand, Sandy Clay, Sandy Mud, Sandy Silt, Clay, Mud, Silt, Gravel, Sandy Gravel, Gravelly Sand, or Slightly Gravelly Sand |
| Mean Grain Size (µm) | Mean grain size, in microns (after Folk and Ward, 1957) |
| Sorting (µm) | Sample sorting - the standard deviation of the grain size distribution, in microns (after Folk and Ward, 1957) |
| Skewness (µm) | Sample skewness - deviation of the grain size distribution from symmetrical, in microns (after Folk and Ward, 1957) |
| Kurtosis (µm) | Sample kurtosis - degree of curvature near the mode of the grain size distribution, in microns (after Folk and Ward, 1957) |
| Mean Grain Size (ɸ) | Mean grain size, in phi units (after Folk and Ward, 1957) |
| Sorting (ɸ) | Sample sorting - the standard deviation of the grain size distribution, in phi units (after Folk and Ward, 1957) |
| Skewness (ɸ) | Sample skewness - deviation of the grain size distribution from symmetrical, in phi units (after Folk and Ward, 1957) |
| Kurtosis (ɸ) | Sample kurtosis - degree of curvature near the mode of the grain size distribution, in phi units (after Folk and Ward, 1957) |
| Mean Grain Size (Descriptive) | Physical description of mean grain size (after Folk and Ward, 1957): Clay, Very Fine Silt, Fine Silt, Medium Silt, Coarse Silt, Very Coarse Silt, Very Fine Sand, Fine Sand, Medium Sand, Coarse Sand, Very Coarse Sand, or Fine Gravel |
| Sorting (Descriptive) | Physical description of sample sorting (after Folk and Ward, 1957): Very Well Sorted, Well Sorted, Moderately Well Sorted, Moderately Sorted, Poorly Sorted, Very Poorly Sorted, or Extremely Poorly Sorted |
| Skewness (Descriptive) | Physical description of sample skewness (after Folk and Ward, 1957): Very Fine Skewed, Fine Skewed, Symmetrical, Coarse Skewed, or Very Coarse Skewed |
| Kurtosis (Descriptive) | Physical description of sample kurtosis (after Folk and Ward, 1957): Very Platykurtic, Platykurtic, Mesokurtic, Leptokurtic, Very Leptokurtic, or Extremely Leptokurtic |
| D10 (µm) | Particle diameter representing the 10% cumulative percentile value (10% of the particles in the sediment sample are finer than the D10 grain size), in microns |
| D50 (µm) | Particle diameter representing the 50% cumulative percentile value (50% of the particles in the sediment sample are finer than the D50 grain size), in microns |
| D90 (µm) | Particle diameter representing the 90% cumulative percentile value (90% of the particles in the sediment sample are finer than the D90 grain size), in microns |
| Gravel (%) | Total gravel (>2 millimeter diameter) fraction of the sediment, in percent |
| Sand (%) | Total sand fraction of the sediment sample, in percent |
| Mud (%) | Total mud (silt and clay; <63 micron diameter) fraction of the sediment sample, in percent |
| Very Coarse Gravel (%) | Fraction of the sediment sample that is very coarse gravel (32 to 64 mm diameter, or -5 to -6 phi), in percent |
| Coarse Gravel (%) | Fraction of the sediment sample that is coarse gravel; (16 to 32 mm diameter, or -4 to -5 phi), in percent |
| Medium Gravel (%) | Fraction of the sediment sample that is medium gravel (8 to 16 mm diameter, or -3 to -4 phi), in percent |
| Fine Gravel (%) | Fraction of the sediment sample that is fine gravel (4 to 8 mm diameter, or -2 to -3 phi), in percent |
| Very Fine Gravel (%) | Fraction of the sediment sample that is very fine gravel (2 to 4 mm diameter, or -1 to -2 phi), in percent |
| Very Coarse Sand (%) | Fraction of the sediment sample that is very coarse sand (1 to 2 millimeter diameter, or -1 to 0 phi), in percent |
| Coarse Sand (%) | Fraction of the sediment sample that is coarse sand (500 microns to 1 millimeter diameter, or 0 to 1 phi), in percent |
| Medium Sand (%) | Fraction of the sediment sample that is medium sand (250 to 500 micron diameter, or 1 to 2 phi), in percent |
| Fine Sand (%) | Fraction of the sediment sample that is fine sand (125 to 250 micron diameter, or 2 to 3 phi), in percent |
| Very Fine Sand (%) | Fraction of the sediment sample that is very fine sand (63 to 125 micron diameter, or 3 to 4 phi), in percent |

Folk, R.L., 1954, The distinction between grain size and mineral composition in sedimentary rock nomenclature: Journal of Geology, v. 62, no. 4, p. 344-359, https://doi.org/10.1086/626171.

Folk, R.L., and Ward, W.C., 1957, Brazos River bar [Texas]—A study in the significance of grain size parameters: Journal of Sedimentary Petrology, v. 27, no. 1, p. 3–26, https://doi.org/10.1306/74D70646-2B21-11D7-8648000102C1865D.